Instructions:
In a previous lab, 192.168.1.0/24 was broken up to support the subnets in this lab.

Now you need to subnet the network to extend and conserve IP addresses:

1) Break up 192.168.1.64/26 to support as many subnets as possible with 8 hosts per subnet.

2) Allocate the first new subnet to site 3.

3) Manually configure all devices in the subnet.

- Router with last IP address in subnet

- Hosts from first IP address

4) Subnet the last new subnet you got from 192.168.1.64/26 with /30 masks

and then allocate the subnets to the serial links. Configure the routers appropriately.

5) Verify that PCs can access cisco.com and facebook.com using their browsers

Solution:
1) Allocated Domain: 192.168.1.64 /26 – 255.255.255.192
 Range: 192.168.1.64 ->192.168.1.127
2^4 <= 8+2, 4 bits needed, 32 – 4 = 28
11111 1111.1111 1111.1111 1111.11110000
255	         . 255	       . 255     	    . (128 + 64 + 32 + 16)
Network block jump, 256-240 = 16
Subnet’d Domain: 192.168.1.64/28 – 255.255.255.240
2)  1st Subnet: 192.168.1.64/28 – Usable Host Range: 192.168.1.65 -> 192.168.1.78
      2nd Subnet:192.168.1.80/28 – Usable Host Range: 192.168.1.81 -> 192.168.1.94
      3rd Subnet: 192.168.1.96/28 – Usable Host Range: 192.168.1.97 -> 192.168.1.110
4) 4th Subnet: 192.168.1.112/28
     Subnet’d Domain: 192.168.1.112/30 (255.255.255.252) – Network Block size: 256-252 =4
     Viable Host Range: 192.168.1.113 -> 192.168.1.114
    5th Subnet: 192.168.1.116/30
    VHR: 192.168.1.117->192.168.1.118
    6th Subnet: 192.168.1.120/30 
    VHR: 192.168.1.121 -> 192.168.1.122
    7th Subnet: 192.168.1.124/30
    VHR: 192.168.1.125 -> 192.168.1.126